
TENDER / MATERIAL SUBMITTAL INFORMATION
	
	

	Project Name
	______________________________________________  

	Client
	______________________________________________

	Contractor
	______________________________________________

	Project Location / Site
	______________________________________________

	Tender Reference No.
	______________________________________________

	Specification Section
	______________________________________________

	Submission Date
	______________________________________________

	Prepared By
	______________________________________________


	Description
	Model Name
	Gap Size
	Image

	Supply and apply Pinnacle PRI aluminum roof joint cover system designed for seismic applications, providing a durable and watertight transition at roof-to-wall interfaces where exterior wall seals are used. The PRI features a robust 2 mm thick aluminum cover-plate design with exposed metal components and an integrated secondary EPDM water barrier for enhanced weather protection. The system includes Duroflex material rated at 80 Shore A, compliant with ASTM D2000, ensuring long-term flexibility and resistance to environmental stresses. Manufactured using high-quality extruded aluminum alloys 6063-T5/T6/T52; 6061-T5/T6/T51; 6105-T5/T6; 6005-T5; and 6005A-T5/T61 in accordance with ASTM B221, the PRI-200 ensures structural reliability under seismic movement conditions. Sheet components are produced using AA1100/1200, 19000 M, 19000 H2, 31000 H2, and 31000 H3, meeting the requirements of IS 737 and ASTM B209 for superior corrosion resistance and formability. The system is designed to accommodate ±50% movement, ensuring dependable performance under typical seismic and thermal building movement.

	 PRI & PRIW

	50-250mm


	   [image: image1.png]





Source: Pinnacle Solutions, 201/A, Arihant Royale, Shastri Nagar,Lal Bahadur Road, Mulund (W), Mumbai – 400080, Web: www.pinnaclesolutions.inTel.: +91-8108168086,E-mail: info@pinnaclesolutions.in

