
TENDER / MATERIAL SUBMITTAL INFORMATION
	
	

	Project Name
	______________________________________________

	Client
	______________________________________________

	Contractor
	______________________________________________

	Project Location / Site
	______________________________________________

	Tender Reference No.
	______________________________________________

	Specification Section
	______________________________________________

	Submission Date
	______________________________________________

	Prepared By
	______________________________________________


	Description
	Model Name
	Gap Size
	Image

	Supply and apply Pinnacle PFI Series flush-mounted metal floor-to-floor joint cover system designed for both interior and exterior applications requiring a durable, flexible, and low-maintenance solution. The system features a self-centering aluminum center plate that accommodates thermal and seismic activity while maintaining a flush walking surface. Suitable for gap sizes ranging from 100 mm to 350 mm, the PFI system provides reliable movement capability across medium to large expansion joints. Manufactured using high-quality extruded aluminum alloys including 6063-T5/T6/T52; 6061-T5/T6/T51; 6105-T5/T6; 6005-T5; and 6005A-T5/T61, all in compliance with ASTM B221, the system ensures long-term durability and structural performance. Sheet components are produced from AA1100/1200, 19000 M, 19000 H2, 31000 H2, and 31000 H3, meeting IS 737 and ASTM B209 standards. As a cover plate–type system, the PFI is mechanically anchored and supports standard-duty loading of 500 lbs. An EPDM sheet moisture barrier is integrated for enhanced protection, and an aluminum spherical turn bar ensures secure alignment and installation integrity. The PFI Series is engineered to accommodate ±30% movement, providing reliable performance under thermal and seismic building conditions.

	PFI & PFIW

	100-350mm

	[image: image1.png]o

+
.
.

R

. .
7 X






Source: Pinnacle Solutions, 201/A, Arihant Royale, Shastri Nagar,Lal Bahadur Road, Mulund (W), Mumbai – 400080, Web: www.pinnaclesolutions.inTel.: +91-8108168086,E-mail: info@pinnaclesolutions.in

